| SECTION 1: Identification of the substance/mixture and of the company/undertaking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  | ### Product identifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Name: |  |  |  | NnF CERAM® Al₂O₃ |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Chemical name |  |  |  | Aluminium oxide |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CAS number |  |  |  | 1344-28-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | EC number: |  |  |  | 215-691-6 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Form: |  |  |  | Nanoform |  |  |  |  |  |  |  |  |  |  |  |
| 1.2 |  | ### Relevant identified uses of the substance or mixture and uses advised against |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Identified uses: |  |  |  | catalyst carrier, reinforcement of ceramics (for corundum phase), abrasive material |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uses advised against: |  |  |  | use the mixture only for the purposes specified by the manufacturer. Otherwise, the user may be exposed to unpredictable risks. |  |  |  |  |  |  |  |  |  |  |  |
| 1.3 |  | Details of the supplier of the safety data sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Supplier: |  |  |  | PARDAM NANO4FIBERS s.r.o. Žižkova 2759 Roudnice nad Labem 41301 Czech Republic |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Telehone: |  |  |  | +420565552610 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Responsible person: |  |  |  | ENVI GROUP s.r.o., Příčná 2186, 347 01 Tachov, Czech Republic tel.: +420606638325, email: info@envigroup.cz |  |  |  |  |  |  |  |  |  |  |  |
| 1.4 |  | Emergency telephone number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Call your national emergency center. Universal emergency telephone number EU: 112 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 2: Hazards identification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 |  | Classification of the substance or mixture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Classification 1272/2008/WE: |  |  |  |  |  |  |  |  | The substance is not classified as dangerous. |  |  |  |  |  |  |
|  |  | Dangerous health effects: |  |  |  |  |  |  |  |  | may irritate the respiratory tract, eyes and skin |  |  |  |  |  |  |
|  |  | Dangerous effects on the environment: |  |  |  |  |  |  |  |  | not classified |  |  |  |  |  |  |
|  |  | Dangerous physical-chemical effects: |  |  |  |  |  |  |  |  | not classified |  |  |  |  |  |  |
| 2.2 |  | Label elements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Contains: |  |  |  |  |  |  |  | Aluminium oxide, CAS 1344-28-1 |  |  |  |  |  |  |  |
|  |  | Hazard pictogram: |  |  |  |  |  |  |  | not required |  |  |  |  |  |  |  |
|  |  | Signal word: |  |  |  |  |  |  |  | not required |  |  |  |  |  |  |  |
|  |  | Hazard statement: |  |  |  |  |  |  |  | not required |  |  |  |  |  |  |  |
|  |  | Precautionary statements: |  |  |  |  |  |  |  | not required |  |  |  |  |  |  |  |
| 2.3 |  | Other hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The substance is not assessed as PBT or vPvB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 3: Composition/information on ingredients |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 |  | Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Substance identifier |  |  |  |  |  |  |  | CAS number EC number Index number Registration number |  |  |  | Content (%) |  |  |  | Classification 1272/2008 |  |
| Aluminium oxide |  |  |  |  |  |  |  | 1344-28-1 215-691-6 - - |  |  |  | 99 |  |  |  | Not classified |  |
| 3.2 |  | Mixtures not applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full text of hazard statement is specified in chapter 16. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 4: First aid measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Description of first aid measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | First aid personnel must take care on their own safety. In case of accident or if you feel unwell, or in case of the occurrence of any symptoms or doubts, consult your health condition with a doctor and provide information from this safety data sheet. In case of unconsciousness place the affected person into the stabilized position on the side and do not administer any oral products. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Inhalation: |  |  |  | Provide fresh air to the affected person. If the problem persists, see a doctor. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Skin contact: |  |  |  | Remove contaminated clothing, wash contaminated skin thoroughly with water. If the problem persists, see a doctor. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Eye contact: |  |  |  | Rinse open eyes immediately with water for about 15 minutes, do not rinse with a strong stream of water - risk of corneal injury. If the problem persists, see a doctor. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ingestion: |  |  |  | Do not induce vomiting. Rinse mouth with water. Consult a doctor. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2 | Most important symptoms and effects, both acute and delayed May irritate the respiratory tract |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.3 | Indication of any immediate medical attention and special treatment needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Special funds are not intended. Treatment is symptomatic. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 5: Firefighting measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Extinguishing media |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Suitable extinguishing media: |  |  |  |  |  |  |  | fire extinguishing agents adapt to the surrounding fire |  |  |  |  |  |  |  |  |
|  | Unsuitable extinguishing media: |  |  |  |  |  |  |  | strong stream of water |  |  |  |  |  |  |  |  |
| 5.2 | Special hazards arising from the substance or mixture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | In case of fire, carbon dioxide, carbon monoxide and other toxic gases may be formed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.3 | Advice for firefighters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Use suitable respiratory protective device. Collect contaminated fire fighting water separately. It must not enter the sewage system. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 6: Accidental release measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Personal precautions, protective equipment and emergency procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Restrict unauthorized access to the accident area until the accident is rectified. Ensure adequate ventilation. Avoid contact with skin and eyes. Avoid dust generation. Avoid inhaling dust. Use personal protective equipment (face masks FP2 or FP3, gloves). Follow the rules and regulations of work safety when working with chemical products. Use personal protective equipment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2 | Environmental precautions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Do not allow to enter sewers / surface or ground water. If the product leaks into water or sewage system inform appropriate authorities. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3 | Methods and material for containment and cleaning up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Collect and dispose of without creating dust (wet wipping). Store in suitable closed containers for disposal. Dispose of collected material in accordance with local regulations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4 | Reference to other sections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Section 8 and 13. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 7: Handling and storage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Precautions for safe handling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Avoid dust formation. Ensure good ventilation and extraction at the workplace. Do not breathe dust. Avoid contact with eyes and skin. For personal protection see section 8. Do not drink, eat or smoke while using this product. Wash hands after use. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2 | Conditions for safe storage, including any incompatibilities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Store in closed containers in a dry, cool and well-ventilated place. Do not eat, drink or smoke in areas where the product is stored and used. Strongly hygroscopic. Protect from direct sunlight. Do not store with food, drinks and feed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3 | Specific end use(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Information not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 8: Exposure controls/personal protection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Control parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Occupational exposure limit values: not available DNEL - information not available PNEC - information not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2 | ### Exposure controls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ensure adequate ventilation. Ensure that persons using personal protective equipment work with the product. After work, wash your hands thoroughly with soap and water. Do not eat, drink or smoke while working. The monitoring procedure for the content of substances in the workplace air and the specification of protective equipment shall be determined by the worker responsible for occupational safety and health protection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Individual protection measures, such as personal protective equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Respiratory protection: |  |  |  | In case of insufficient ventilation, use a respirator type P1 or ABEK-P2 (EN 143) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Eye protection: |  |  |  | Safety goggles (EN 166) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hand protection: |  |  |  | Wear protective gloves (EN 374). The glove material has to be impermeable and resistant to the product. Consult the manufacturer for the choice of gloves. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Skin protection: |  |  |  | Suitable work clothes and shoes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Environmental exposure controls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Prevent leakage to the sewage system, surface waters, underground waters and soil. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 9: Physical and chemical properties |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1 | Information on basic physical and chemical properties |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Physical state |  |  |  |  |  |  |  |  |  |  |  |  |  | solid: filaments |  |  |
|  | Colour |  |  |  |  |  |  |  |  |  |  |  |  |  | white |  |  |
|  | Color intensity |  |  |  |  |  |  |  |  |  |  |  |  |  | light |  |  |
|  | Odour |  |  |  |  |  |  |  |  |  |  |  |  |  | without fragrance (nanoform) |  |  |
|  | Melting point/freezing point (°C): |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 - 2054 °C (nanoform) |  |  |
|  | Boiling point or initial boiling point and boiling range (°C): |  |  |  |  |  |  |  |  |  |  |  |  |  | 2977 °C |  |  |
|  | Flammability |  |  |  |  |  |  |  |  |  |  |  |  |  | Information not available |  |  |
|  | Lower and upper explosion limit: upper limit (% vol.): lower limit (% vol.): |  |  |  |  |  |  |  |  |  |  |  |  |  | Information not available |  |  |
|  | Flash point (°C): |  |  |  |  |  |  |  |  |  |  |  |  |  | Information not available |  |  |
|  | Auto-ignition temperature |  |  |  |  |  |  |  |  |  |  |  |  |  | Information not available |  |  |
|  | Decomposition temperature |  |  |  |  |  |  |  |  |  |  |  |  |  | Information not available |  |  |
|  | pH |  |  |  |  |  |  |  |  |  |  |  |  |  | non-soluble (in water) |  |  |
|  | Kinematic viscosity |  |  |  |  |  |  |  |  |  |  |  |  |  | Information not available |  |  |
|  | Solubility |  |  |  |  |  |  |  |  |  |  |  |  |  | insoluble |  |  |
|  | Partition coefficient n-octanol/water (log value) |  |  |  |  |  |  |  |  |  |  |  |  |  | Information not available |  |  |
|  | Vapour pressure |  |  |  |  |  |  |  |  |  |  |  |  |  | 1hPa at 2158 °C (nanoform) |  |  |
|  | Density and/or relative density |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,679 g/cm³ at 20 °C (nanoform) |  |  |
|  | Relative vapour density |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.5 - 4 (nanoform) |  |  |
|  | Particle characteristics |  |  |  |  |  |  |  |  |  |  |  | fiber strength 100-800 nm; fiber length 1000 - 10000 nm |  |  |  |  |
| 9.2 | Other information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Information not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 10: Stability and reactivity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Reactivity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | React in acid environements (creates salts) and in alkaline (creates aluminates) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.2 | Chemical stability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The product is stable except for the example given in point 10.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.3 | Possibility of hazardous reactions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Not known. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.4 | Conditions to avoid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Information not available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.5 | Incompatible materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Strong oxidizing agents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.6 | Hazardous decomposition products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | They do not arise spontaneously. In case of fire, carbon dioxide and other toxic gases may be formed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 11: Toxicological information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 | Information on hazard classes as defined in Regulation (EC) No 1272/2008 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | a) Acute toxicity Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b) |  | Skin corrosion/irritation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c) |  | Serious eye damage/irritation Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d) |  | Respiratory or skin sensitisation Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | e) |  | Germ cell mutagenicity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | f) |  | Carcinogenicity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | g) |  | Reproductive toxicity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | h) STOT-single exposure Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i) STOT-repeated exposure Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | j) Aspiration hazard Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.2 | Information on other hazards May irritate the respiratory tract, eyes and skin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 12: Ecological information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 | Toxicity The mixture is not classified as dangerous for the aquatic environment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.2 | Persistence and degradability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Information not available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.3 | ### Bioaccumulative potential |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Information not available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.4 | Mobility in soil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Information is not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.5 | Results of PBT and vPvB assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The substance is not assessed as PBT or vPvB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.6 | Endocrine disrupting properties They are not. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.7 | Other adverse effects |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Do not allow to enter drains, surface or ground water. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 13: Disposal considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 |  | ### Waste treatment methods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | a) | Suitable methods for removing the mixture and contaminated packaging: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Hand over the marked waste for disposal to a specialized company authorized for this activity. It must not be disposed of with municipal waste. Dispose of waste in accordance with the Waste Act. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | b) | Specify the physical/chemical properties that may affect waste treatment options |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | information is not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | c) | Discourage sewage disposal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | information is not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | d) | Iidentify, where appropriate, any special precautions for any recommended waste treatment option |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | information is not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 14: Transport information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The product is not dangerous for transport |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 |  | UN number or ID number: The product is not dangerous for transport |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.2 |  | UN proper shipping name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Ground transportation ADR |  |  |  |  | - CORROSIVE SUBSTANCE, SOLID, J.N. (sodium oxide) CORROSIVE SOLID, N.O.S. (Disodium oxide) CORROSIVE SOLID, N.O.S. (Disodium oxide) |  |  |  |  |  |  |  |  |  |  |
|  |  | Railway transportation RID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Naval transport IMDG: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Air transport ICAO/IATA: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.3 |  | Transport hazard class(es) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ADR |  |  |  | RID |  |  |  |  |  |  |  | IMDG: |  |  | ICAO/IATA: |
|  |  | - |  |  |  | - |  |  |  |  |  |  |  | - |  |  | - |
| 14.4 |  | Packing group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ADR |  |  |  | RID |  |  |  |  |  |  |  | IMDG: |  |  | ICAO/IATA: |
|  |  | - |  |  |  | - |  |  |  |  |  |  |  | - |  |  | - |
| 14.5 |  | ### Environmental hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Substance is not hazardous to the environment during transport. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.6 |  | Special precautions for user |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The product is not dangerous for transport |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.7 |  | Maritime transport in bulk according to IMO instruments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Information is not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 15: Regulatory information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.1 |  | Safety, health and environmental regulations/legislation specific for the substance or mixture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Regulation (EC) No 1907/2006 (REACH), Regulation (EC) No 1272/2008 (CLP), Commission Regulation (EU) 2020/878, the Waste Act as amended |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.2 |  | Chemical safety assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 16: Other information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | a) | Changes made to the safety data sheet on 23.10.2023: translation of the safety data sheet according to Regulation (EC) No. 878/2020 and Regulation (EC) No. 1272/2008 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | b) | List of abbreviations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DNEL | Derived No Effect Level |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PNEC | Predicted No Effect Concentration |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CLP | Regulation (EC) No 1272/2008 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | REACH | Regulation (EC) No 1907/2006 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | IMDG | International Code of Dangerous Goods |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | IATA | International Air Transport Association |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ICAO | International Civil Aviation Organization |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | RID | Regulation on the international transport of dangerous goods by rail |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Einecs | European Inventory of Existing Commercial Chemical Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CAS | Chemical Abstracts Service |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PBT | persistent, bioaccumulative and toxic substance |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | vPvB | very persistent a very bioaccumulation substance |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | SDS | Safety data sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | c) | Sources of information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | State legislation, original manufacturer's safety data sheets, ECHA website. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | d) | List of hazard statements: not used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | e) | Instructions for training |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Training of work safety for handling with chemicals. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | f) | Other information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | The information in this safety data sheet correspond to the best knowledge of the product at the time of publication. This information is used for safer handling, storage, transportation and disposal of the product. This information does not guarantee the quality of the product. This information applies only to this material and is not valid if the product is used in combination with other materials or in a way which is not listed in this SDS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |