| SECTION 1: Identification of the substance/mixture and of the company/undertaking |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  | ### Product identifier |  |  |  |  |  |  |  |  |
|  |  | Name: |  | NnF CERAM® ZrO₂ |  |  |  |  |  |  |
|  |  | Chemical name: |  | Zirconium dioxide |  |  |  |  |  |  |
|  |  | CAS number: |  | 1314-23-4 |  |  |  |  |  |  |
|  |  | EC number: |  | 215-227-2 |  |  |  |  |  |  |
|  |  | Form: |  | Nanoform |  |  |  |  |  |  |
| 1.2 |  | ### Relevant identified uses of the substance or mixture and uses advised against |  |  |  |  |  |  |  |  |
|  |  | Identified uses: |  | ceramics reinforcement, oxygen sensors, thermal barriers |  |  |  |  |  |  |
|  |  | Uses advised against: |  | use the mixture only for the purposes specified by the manufacturer. Otherwise, the user may be exposed to unpredictable risks. |  |  |  |  |  |  |
| 1.3 |  | Details of the supplier of the safety data sheet |  |  |  |  |  |  |  |  |
|  |  | Supplier: |  | PARDAM NANO4FIBERS s.r.o. Žižkova 2759 Roudnice nad Labem 41301 Czech Republic |  |  |  |  |  |  |
|  |  | Telehone: |  | +420565552610 |  |  |  |  |  |  |
|  |  | Responsible person: |  | ENVI GROUP s.r.o., Příčná 2186, 347 01 Tachov, Czech Republic tel.: +420606638325, email: info@envigroup.cz |  |  |  |  |  |  |
| 1.4 |  | Emergency telephone number |  |  |  |  |  |  |  |  |
|  |  | Call your national emergency center. Universal emergency telephone number EU: 112 |  |  |  |  |  |  |  |  |
| SECTION 2: Hazards identification |  |  |  |  |  |  |  |  |  |  |
| 2.1 |  | Classification of the substance or mixture |  |  |  |  |  |  |  |  |
|  |  | Classification 1272/2008/WE: |  |  |  |  |  | The substance is classified as dangerous. |  |  |
|  |  |  |  |  |  |  |  | Skin Irrit. 2, H315 Eye Irrit. 2, H319 |  |  |
|  |  | Dangerous health effects: |  |  |  |  |  | Causes skin and eye irritation |  |  |
|  |  | Dangerous effects on the environment: |  |  |  |  |  | not classified |  |  |
|  |  | Dangerous physical-chemical effects: |  |  |  |  |  | not classified |  |  |
| 2.2 |  | Label elements |  |  |  |  |  |  |  |  |
|  |  | Contains: |  |  |  |  | Zirconium dioxide, CAS 1314-23-4 |  |  |  |
|  |  | Hazard pictogram: |  |  |  |  |  |  |  |  |
|  |  | Signal word: |  |  |  |  | Warning |  |  |  |
|  |  | Hazard statement: |  |  |  |  | H315 Causes skin irritation. H319 Causes serious eye irritation. |  |  |  |
|  |  | Precautionary statements: |  |  |  |  | P264 Wash hands thoroughly after handling. P280 Wear protective gloves/protective clothing/eye protection. P302+P352 IF ON SKIN: Wash with plenty of water. P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P314 Get medical advice/attention if you feel unwell. P501 Dispose of contents/container in accordance with local regulations. |  |  |  |
| 2.3 |  | Other hazards |  |  |  |  |  |  |  |  |
|  |  | The substance is not assessed as PBT or vPvB |  |  |  |  |  |  |  |  |
| SECTION 3: Composition/information on ingredients |  |  |  |  |  |  |  |  |  |  |
| 3.1 |  | Substances |  |  |  |  |  |  |  |  |
| Substance identifier |  |  |  |  | CAS number EC number Index number Registration number |  |  |  | Content (%) | Classification 1272/2008 |
| Zirconium dioxide |  |  |  |  | 1314-23-4 215-227-2 - - |  |  |  | ≤ 100 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 |
| 3.2 |  | Mixtures Does not apply. |  |  |  |  |  |  |  |  |
| Full text of hazard statement is specified in chapter 16. |  |  |  |  |  |  |  |  |  |  |
| SECTION 4: First aid measures |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Description of first aid measures |  |  |  |  |  |  |  |  |  |
|  | First aid personnel must take care on their own safety. In case of accident or if you feel unwell, or in case of the occurrence of any symptoms or doubts, consult your health condition with a doctor and provide information from this safety data sheet. In case of unconsciousness place the affected person into the stabilized position on the side and do not administer any oral products. |  |  |  |  |  |  |  |  |  |
|  | Inhalation: |  | In case of inhalation, remove the person to fresh air. If the problem persists, see a doctor. |  |  |  |  |  |  |  |
|  | Skin contact: |  | Remove contaminated clothing, wash contaminated skin thoroughly with soap and water. If the problem persists, see a doctor. |  |  |  |  |  |  |  |
|  | Eye contact: |  | Rinse open eyes immediately with water for about 15 minutes, do not rinse with a strong stream of water - risk of corneal injury. Consult a doctor. |  |  |  |  |  |  |  |
|  | Ingestion: |  | Do not induce vomiting. Rinse mouth with water. Consult a doctor. |  |  |  |  |  |  |  |
| 4.2 | Most important symptoms and effects, both acute and delayed May irritate the respiratory tract. Causes skin and eye irritation. |  |  |  |  |  |  |  |  |  |
| 4.3 | Indication of any immediate medical attention and special treatment needed |  |  |  |  |  |  |  |  |  |
|  | Special funds are not intended. Treatment is symptomatic. |  |  |  |  |  |  |  |  |  |
| SECTION 5: Firefighting measures |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Extinguishing media |  |  |  |  |  |  |  |  |  |
|  | Suitable extinguishing media: |  |  |  |  | Water mist, CO2, extinguishing powder, alcohol-resistant foam |  |  |  |  |
|  | Unsuitable extinguishing media: |  |  |  |  | fire extinguishing agents adapt to the surrounding fire |  |  |  |  |
| 5.2 | Special hazards arising from the substance or mixture |  |  |  |  |  |  |  |  |  |
|  | In case of fire, carbon dioxide, carbon monoxide and other dangerous gases may be formed. |  |  |  |  |  |  |  |  |  |
| 5.3 | Advice for firefighters |  |  |  |  |  |  |  |  |  |
|  | Use complete protective equipment. |  |  |  |  |  |  |  |  |  |
| SECTION 6: Accidental release measures |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Personal precautions, protective equipment and emergency procedures |  |  |  |  |  |  |  |  |  |
|  | Restrict unauthorized access to the accident area until the accident is rectified. Ensure adequate ventilation. Avoid contact with skin and eyes. Avoid dust generation. Avoid dust inhalation. Follow the rules and regulations of work safety when working with chemical products. Use personal protective equipment (face masks FP2 or FP3, gloves). |  |  |  |  |  |  |  |  |  |
| 6.2 | Environmental precautions |  |  |  |  |  |  |  |  |  |
|  | Do not allow to enter sewers / surface or ground water. If the product leaks into water or sewage system inform appropriate authorities. |  |  |  |  |  |  |  |  |  |
| 6.3 | Methods and material for containment and cleaning up |  |  |  |  |  |  |  |  |  |
|  | Collect and dispose of without creating dust (wet wipping). Store in suitable closed containers for disposal. Dispose of collected material in accordance with local regulations. |  |  |  |  |  |  |  |  |  |
| 6.4 | Reference to other sections |  |  |  |  |  |  |  |  |  |
|  | Section 8 and 13. |  |  |  |  |  |  |  |  |  |

| SECTION 7: Handling and storage |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1 | Precautions for safe handling |  |  |  |  |  |  |  |  |  |
|  | Avoid dust formation. Ensure good ventilation and extraction at the workplace. Do not breathe dust. Avoid contact with eyes and skin. For personal protection see section 8. Do not drink, eat or smoke while using this product. Wash hands after use. |  |  |  |  |  |  |  |  |  |
| 7.2 | Conditions for safe storage, including any incompatibilities |  |  |  |  |  |  |  |  |  |
|  | Store in closed containers in a dry, cool and well-ventilated place. Do not eat, drink or smoke in areas where the product is stored and used. Strongly hygroscopic. Protect from direct sunlight. Do not store with food, drinks and feed. |  |  |  |  |  |  |  |  |  |
| 7.3 | Specific end use(s) |  |  |  |  |  |  |  |  |  |
|  | Information not available |  |  |  |  |  |  |  |  |  |
| SECTION 8: Exposure controls/personal protection |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Control parameters |  |  |  |  |  |  |  |  |  |
|  | Occupational exposure limit values: not available DNEL - information not available PNEC - information not available |  |  |  |  |  |  |  |  |  |
| 8.2 | ### Exposure controls |  |  |  |  |  |  |  |  |  |
|  | Ensure adequate ventilation. Ensure that persons using personal protective equipment work with the product. After work, wash your hands thoroughly with soap and water. Do not eat, drink or smoke while working. The monitoring procedure for the content of substances in the workplace air and the specification of protective equipment shall be determined by the worker responsible for occupational safety and health protection. |  |  |  |  |  |  |  |  |  |
|  | Individual protection measures, such as personal protective equipment |  |  |  |  |  |  |  |  |  |
|  | Respiratory protection: |  |  |  | In case of insufficient ventilation, use a respirator type P1 or ABEK-P2 (EN 143) |  |  |  |  |  |
|  | Eye protection: |  |  |  | Safety goggles (EN 166) |  |  |  |  |  |
|  | Hand protection: |  |  |  | Wear protective gloves (EN 374). The glove material has to be impermeable and resistant to the product. Consult the manufacturer for the choice of gloves. |  |  |  |  |  |
|  | Skin protection: |  |  |  | Suitable work clothes and shoes |  |  |  |  |  |
|  | Environmental exposure controls |  |  |  |  |  |  |  |  |  |
|  | Prevent leakage to the sewage system, surface waters, underground waters and soil. |  |  |  |  |  |  |  |  |  |
| SECTION 9: Physical and chemical properties |  |  |  |  |  |  |  |  |  |  |
| 9.1 | Information on basic physical and chemical properties |  |  |  |  |  |  |  |  |  |
|  | Physical state |  |  |  |  |  |  |  | Solid (filaments) |  |
|  | Colour |  |  |  |  |  |  |  | White |  |
|  | Colour intensity |  |  |  |  |  |  |  | Light |  |
|  | Odour |  |  |  |  |  |  |  | without fragrance (nanoform) |  |
|  | Melting point/freezing point (°C): |  |  |  |  |  |  |  | 2680 - 2710 °C (101.325 Pa nanoform) |  |
|  | Boiling point or initial boiling point and boiling range (°C): |  |  |  |  |  |  |  | data not available |  |
|  | Flammability |  |  |  |  |  |  |  | non-inflammable (nanoform) |  |
|  | Lower and upper explosion limit: upper limit (% vol.): lower limit (% vol.): |  |  |  |  |  |  |  | data not available |  |
|  | Flash point (°C): |  |  |  |  |  |  |  | data not available |  |
|  | Auto-ignition temperature |  |  |  |  |  |  |  | >400°C (nanoform) |  |
|  | Decomposition temperature |  |  |  |  |  |  |  | data not available |  |
|  | pH |  |  |  |  |  |  |  | Non-soluble (in water) |  |
|  | Kinematic viscosity |  |  |  |  |  |  |  | data not available |  |
|  | Solubility |  |  |  |  |  |  |  | data not available |  |
|  | Partition coefficient n-octanol/water (log value) |  |  |  |  |  |  |  | data not available |  |
|  | Vapour pressure |  |  |  |  |  |  |  | data not available |  |
|  | Density |  |  |  |  |  |  |  | 5,77 g/cm³ at 20 °C (nanoform) |  |
|  | Relative vapour density |  |  |  |  |  |  |  | data not available |  |
|  | Particle characteristics |  |  |  |  |  |  |  | powder (nanoform) |  |
| 9.2 | Other information |  |  |  |  |  |  |  |  |  |
|  | data not available |  |  |  |  |  |  |  |  |  |
| SECTION 10: Stability and reactivity |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Reactivity |  |  |  |  |  |  |  |  |  |
|  | Not readily reactive in neither acidic nor alkaline media (i.e. extremely stable compound) |  |  |  |  |  |  |  |  |  |
| 10.2 | Chemical stability |  |  |  |  |  |  |  |  |  |
|  | The produkt is stable under normal ambient conditions and anticipated temperature and pressure conditions during storage and handling. |  |  |  |  |  |  |  |  |  |
| 10.3 | Possibility of hazardous reactions |  |  |  |  |  |  |  |  |  |
|  | Not available |  |  |  |  |  |  |  |  |  |
| 10.4 | Conditions to avoid |  |  |  |  |  |  |  |  |  |
|  | Not available |  |  |  |  |  |  |  |  |  |
| 10.5 | Incompatible materials |  |  |  |  |  |  |  |  |  |
|  | Strong oxidizing agents |  |  |  |  |  |  |  |  |  |
| 10.6 | Hazardous decomposition products |  |  |  |  |  |  |  |  |  |
|  | Not known |  |  |  |  |  |  |  |  |  |
| SECTION 11: Toxicological information |  |  |  |  |  |  |  |  |  |  |
| 11.1 | Information on hazard classes as defined in Regulation (EC) No 1272/2008 |  |  |  |  |  |  |  |  |  |
|  | a) Acute toxicity Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |
|  | b) |  | Skin corrosion/irritation |  |  |  |  |  |  |  |
|  |  |  | Causes skin irritation. |  |  |  |  |  |  |  |
|  | c) |  | Serious eye damage/irritation Causes serious eye irritation. |  |  |  |  |  |  |  |
|  | d) |  | Respiratory or skin sensitisation Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |
|  | e) |  | Germ cell mutagenicity |  |  |  |  |  |  |  |
|  |  |  | Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |
|  | f) |  | Carcinogenicity |  |  |  |  |  |  |  |
|  |  |  | Suspected of causing cancer |  |  |  |  |  |  |  |
|  | g) |  | Reproductive toxicity |  |  |  |  |  |  |  |
|  |  |  | Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |
|  | h) STOT-single exposure Inhalation may cause respiratory irritation. |  |  |  |  |  |  |  |  |  |
|  | i) STOT-repeated exposure Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |
|  | j) Aspiration hazard Based on available data, the classification criteria are not met. |  |  |  |  |  |  |  |  |  |
| 11.2 | Information on other hazards Information is not available |  |  |  |  |  |  |  |  |  |
| SECTION 12: Ecological information |  |  |  |  |  |  |  |  |  |  |
| 12.1 | Toxicity The substance is not classified as dangerous for the aquatic environment. |  |  |  |  |  |  |  |  |  |
| 12.2 | Persistence and degradability |  |  |  |  |  |  |  |  |  |
|  | Information not available. |  |  |  |  |  |  |  |  |  |
| 12.3 | ### Bioaccumulative potential |  |  |  |  |  |  |  |  |  |
|  | Information not available. |  |  |  |  |  |  |  |  |  |
| 12.4 | Mobility in soil |  |  |  |  |  |  |  |  |  |
|  | Information is not available |  |  |  |  |  |  |  |  |  |
| 12.5 | Results of PBT and vPvB assessment |  |  |  |  |  |  |  |  |  |
|  | The substance is not assessed as PBT or vPvB |  |  |  |  |  |  |  |  |  |
| 12.6 | Endocrine disrupting properties Not known |  |  |  |  |  |  |  |  |  |
| 12.7 | Other adverse effects |  |  |  |  |  |  |  |  |  |
|  | Do not allow to enter drains, surface or ground water. |  |  |  |  |  |  |  |  |  |
| SECTION 13: Disposal considerations |  |  |  |  |  |  |  |  |  |  |
| 13.1 |  | ### Waste treatment methods |  |  |  |  |  |  |  |  |
|  |  | a) | Suitable methods for removing the mixture and contaminated packaging: |  |  |  |  |  |  |  |
|  |  |  | Hand over the marked waste for disposal to a specialized company authorized for this activity. It must not be disposed of with municipal waste. Dispose of waste in accordance with local regulations. |  |  |  |  |  |  |  |
|  |  | b) | Specify the physical/chemical properties that may affect waste treatment options |  |  |  |  |  |  |  |
|  |  |  | information is not available |  |  |  |  |  |  |  |
|  |  | c) | Discourage sewage disposal |  |  |  |  |  |  |  |
|  |  |  | information is not available |  |  |  |  |  |  |  |
|  |  | d) | Iidentify, where appropriate, any special precautions for any recommended waste treatment option |  |  |  |  |  |  |  |
|  |  |  | information is not available |  |  |  |  |  |  |  |
| SECTION 14: Transport information |  |  |  |  |  |  |  |  |  |  |
|  |  | The product is not dangerous for transport |  |  |  |  |  |  |  |  |
| 14.1 |  | UN number or ID number: The product is not dangerous for transport |  |  |  |  |  |  |  |  |
| 14.2 |  | UN proper shipping name |  |  |  |  |  |  |  |  |
|  |  | Ground transportation ADR |  |  |  |  | - |  |  |  |
|  |  | Railway transportation RID |  |  |  |  |  |  |  |  |
|  |  | Naval transport IMDG: |  |  |  |  |  |  |  |  |
|  |  | Air transport ICAO/IATA: |  |  |  |  |  |  |  |  |
| 14.3 |  | Transport hazard class(es) |  |  |  |  |  |  |  |  |
|  |  | ADR |  |  |  | RID |  | IMDG: |  | ICAO/IATA: |
|  |  | - |  |  |  | - |  | - |  | - |
| 14.4 |  | Packing group |  |  |  |  |  |  |  |  |
|  |  | ADR |  |  |  | RID |  | IMDG: |  | ICAO/IATA: |
|  |  | - |  |  |  | - |  | - |  | - |
| 14.5 |  | ### Environmental hazards |  |  |  |  |  |  |  |  |
|  |  | Substance is not hazardous to the environment during transport. |  |  |  |  |  |  |  |  |
| 14.6 |  | Special precautions for user |  |  |  |  |  |  |  |  |
|  |  | The product is not dangerous for transport |  |  |  |  |  |  |  |  |
| 14.7 |  | Maritime transport in bulk according to IMO instruments |  |  |  |  |  |  |  |  |
|  |  | Information is not available |  |  |  |  |  |  |  |  |
| SECTION 15: Regulatory information |  |  |  |  |  |  |  |  |  |  |
| 15.1 |  | Safety, health and environmental regulations/legislation specific for the substance or mixture |  |  |  |  |  |  |  |  |
|  |  | Regulation (EC) No 1907/2006 (REACH), Regulation (EC) No 1272/2008 (CLP), Commission Regulation (EU) 2020/878, the Waste Act as amended |  |  |  |  |  |  |  |  |
| 15.2 |  | Chemical safety assessment |  |  |  |  |  |  |  |  |
|  |  | A chemical safety assessment has not been carried out for this product |  |  |  |  |  |  |  |  |
| SECTION 16: Other information |  |  |  |  |  |  |  |  |  |  |
|  |  | a) | Changes made to the safety data sheet on 18.10.2023: adapt of the safety data sheet according to Regulation (EC) No. 878/2020 of the European Parliament and of the Council and according to Regulation (EC) No. 1272/2008 of the European Parliament and of the Council |  |  |  |  |  |  |  |
|  |  | b) | List of abbreviations |  |  |  |  |  |  |  |
|  |  |  | DNEL | Derived No Effect Level |  |  |  |  |  |  |
|  |  |  | PNEC | Predicted No Effect Concentration |  |  |  |  |  |  |
|  |  |  | CLP | Regulation (EC) No 1272/2008 |  |  |  |  |  |  |
|  |  |  | REACH | Regulation (EC) No 1907/2006 |  |  |  |  |  |  |
|  |  |  | IMDG | International Code of Dangerous Goods |  |  |  |  |  |  |
|  |  |  | IATA | International Air Transport Association |  |  |  |  |  |  |
|  |  |  | ICAO | International Civil Aviation Organization |  |  |  |  |  |  |
|  |  |  | ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |  |  |  |  |  |  |
|  |  |  | RID | Regulation on the international transport of dangerous goods by rail |  |  |  |  |  |  |
|  |  |  | Einecs | European Inventory of Existing Commercial Chemical Substances |  |  |  |  |  |  |
|  |  |  | CAS | Chemical Abstracts Service |  |  |  |  |  |  |
|  |  |  | PBT | persistent, bioaccumulative and toxic substance |  |  |  |  |  |  |
|  |  |  | vPvB | very persistent a very bioaccumulation substance |  |  |  |  |  |  |
|  |  |  | SDS | Safety data sheet |  |  |  |  |  |  |
|  |  |  | Skin Irrit. 2 | Skin corrosion/irritation, category 2 |  |  |  |  |  |  |
|  |  |  | Eye Irrit. 2 | Serious eye damage / eye irritation, category 2 |  |  |  |  |  |  |
|  |  | c) | Sources of information |  |  |  |  |  |  |  |
|  |  |  | State legislation, original manufacturer's safety data sheets, ECHA website. The classification was made by the manufacturer according to the EC classification rules CLP 1272/2008 |  |  |  |  |  |  |  |
|  |  | d) | List of hazard statements: |  |  |  |  |  |  |  |
|  |  |  | H315 | Causes skin irritation. |  |  |  |  |  |  |
|  |  |  | H319 | Causes serious eye irritation. |  |  |  |  |  |  |
|  |  | e) | Instructions for training |  |  |  |  |  |  |  |
|  |  |  | Training of work safety for handling with chemicals. |  |  |  |  |  |  |  |
|  |  | f) | Other information |  |  |  |  |  |  |  |
|  |  |  | The information in this safety data sheet correspond to the best knowledge of the product at the time of publication. This information is used for safer handling, storage, transportation and disposal of the product. This information does not guarantee the quality of the product. This information applies only to this material and is not valid if the product is used in combination with other materials or in a way which is not listed in this SDS. |  |  |  |  |  |  |  |